# Sand Island Container Terminal Freight Analysis

Estimating Commodity Flows for Transportation Planning

## Agenda

| Section | Topics |
| --- | --- |
| Introduction | Project purpose, data challenges |
| Data Sources | FAF 5.7.1, Harbor Master Plan, SICT Wharfage Data |
| Methodology | Steps 1-5: Data collection, mapping, processing, calibration |
| Estimation Models | Three model scenarios, Liquid-Bulk considerations |
| Results & Analysis | Pier proportions, SICT share, top commodities, ranking shifts |
| Conclusion | Key takeaways, limitations & considerations |

## Introduction

### Project Purpose

Estimate types and volumes of freight commodities arriving at Sand Island Container Terminal (SICT) via water for 2024

Support transportation planning and infrastructure assessment for the Sand Island corridor

All inbound cargo at SICT piers must exit via the Sand Island Access Road bridge

Directly applicable to bridge traffic analysis

### Data Challenges

No SICT-specific data: No publicly available dataset with detailed commodity breakdown for SICT

Regional FAF data: Freight Analysis Framework(FAF) covers "Honolulu HI" region, not individual piers

Disaggregation needed: Multiple sources used to estimate SICT's share of regional flows

Limited actual data: SICT wharfage data provides totals but lacks commodity detail

## Data Sources

### Primary Data Inputs

| Source | Description | Use in Analysis |
| --- | --- | --- |
| FAF 5.7.1 Regional | USDOT freight flow data by commodity | Commodity volumes & distributions |
| Honolulu Harbor 2050 Master Plan | HDOT pier capacity data | Pier-specific allocations |
| SICT Wharfage Data | Actual port throughput (FY2025) | Calibration of FAF estimates |

### Key Temporal Assumptions

FAF reference year: Calendar year 2024 estimates

SICT wharfage period: July 2024 - June 2025 (FY2025)

Assumption: One-year wharfage data is representative of CY2024 volumes

## Methodology

### Methodology Overview

Step 1: Pier Operations Data    → Capacity proportions per pier
         ↓
Step 2: Cargo Type Definitions  → 5 categories (Container, RO/RO, Break-Bulk, Liquid-Bulk, Dry-Bulk)
         ↓
Step 3: Commodity Mapping       → 42 SCTG2 codes → Cargo types + containerization %
         ↓
Step 4: Automated Processing    → Filter FAF → Aggregate → Distribute to piers
         ↓
Step 5: SICT Calibration        → Scale FAF to actual port data → 3 output scenarios

### Step 1: Pier Operations Data Collection

Source: Honolulu Harbor 2050 Master Plan (manual extraction)

Output: Pier Operations and Cargo Inventory spreadsheet

| Data Extracted | Description |
| --- | --- |
| Pier identification | All piers in Honolulu Harbor |
| Container capacity (TEUs) | Annual container handling capacity |
| RO/RO capacity | Annual vehicle handling capacity |
| Break-Bulk capacity (Tons) | Annual break-bulk capacity |
| Liquid-Bulk capacity (Bbls) | Annual liquid-bulk capacity |
| Dry-Bulk capacity (Tons) | Annual dry-bulk capacity |

Derived: Pier proportions = Pier Capacity / Total Harbor Capacity

### Step 2: Cargo Type Definitions

Five cargo type categories defined from the Master Plan:

| Cargo Type | Description |
| --- | --- |
| Containers | Standardized shipping containers (TEUs) |
| RO/RO | Roll-on/Roll-off vehicles and wheeled cargo |
| Break-Bulk | Non-containerized general cargo |
| Liquid-Bulk | Petroleum products, chemicals, liquids |
| Dry-Bulk | Coal, aggregates, grain, dry commodities |

### Step 3: Commodity Mapping (SCTG2 to Cargo Types)

Source: Commodity Dictionary spreadsheet

Mapped 42 SCTG2 commodity codes to cargo types

Assigned containerization proportions for each commodity

| Handling Type | Container % | Example Commodities |
| --- | --- | --- |
| Fully Containerized | 100% | Manufactured goods, perishables |
| Fully Non-Containerized | 0% | Grains, aggregates, petroleum |
| Mixed-Mode | 10-90% | Milled grains (90% container, 10% bulk) |

### Step 4: Automated Processing

Python script filters FAF data for Honolulu water-based inbound freight

Filtering Logic:

| Flow Type | Origin | Destination | Mode Filter |
| --- | --- | --- | --- |
| Domestic Water | Other U.S. (not Honolulu) | Honolulu HI | dms_mode = Water |
| Import Direct | Foreign → Honolulu | Honolulu HI | fr_inmode = Water |
| Import Transshipped | Foreign → Other U.S. → Honolulu | Honolulu HI | dms_mode = Water |

Processing Pipeline:

Filter FAF for Honolulu water-based inbound freight (above logic)

Aggregate filtered data by SCTG2 commodity (sum tonnage and value)

Merge cargo type assignments from Commodity Dictionary (Step 3)

Distribute commodity totals to piers based on capacity proportions

### Step 4 (cont.): Distribution Logic

Three handling scenarios based on containerization proportion:

| Scenario | Container % | Distribution Method |
| --- | --- | --- |
| Fully Containerized | 100% | All tonnage distributed by Container pier proportions |
| Fully Non-Containerized | 0% | All tonnage distributed by Bulk/Break-Bulk/RO-RO pier proportions |
| Mixed-Mode | 1-99% | Split: containerized share → Container proportions; remainder → Alternative type proportions |

Example: Milled grain prods. (10,000 tons, 70% containerized, Alternative = Dry-Bulk)

7,000 tons (70%) → distributed to piers by their Container proportions

3,000 tons (30%) → distributed to piers by their Dry-Bulk proportions

### Step 5: SICT Calibration

FAF model estimated ~941,000 tons for SICT piers

Actual port data showed ~4,084,000 tons inbound

Solution: Scale FAF data to match actual throughput while preserving commodity proportions

| Source | Total Tons | Notes |
| --- | --- | --- |
| FAF Model | ~941,000 | Includes ~809K Liquid-Bulk |
| Actual Port Data | 4,084,234 | May exclude Liquid-Bulk |

### Step 5 (cont.): Classification Rules

FAF commodities are mapped to SICT wharfage categories using two dimensions:

| Dimension | Rule |
| --- | --- |
| SICT-Type = "Vehicles" | SCTG2 commodities: "Motorized vehicles" and "Transport equip." |
| SICT-Type = "Cargo Non Vehicles" | All other SCTG2 commodities |
| Containerized = Yes | When cargo_type = "Containers" |
| Containerized = No | When cargo_type ≠ "Containers" (Break-Bulk, Liquid-Bulk, RO/RO, Dry-Bulk) |

Actual Port Data Breakdown (Scaling Targets):

| SICT-Type | Containerized | Actual Port Tons |
| --- | --- | --- |
| Cargo | Yes | 3,958,177 |
| Cargo | No | 8,130 |
| Vehicles | Yes | 27,184 |
| Vehicles | No | 90,742 |

### Step 5 (cont.): Scaling Formula

For each (SICT-Type, Containerized) group:

tonnage_scale = Actual Port Tons / FAF Model Tons
scaled_tons   = FAF_tons  × tonnage_scale
scaled_value  = FAF_value × tonnage_scale

Same scale factor applied to both tonnage and value

Preserves relative commodity proportions within each group

Three output scenarios produced to address Liquid-Bulk uncertainty

## Estimation Models

### Output Sheets in FAF_Hawaii_Region_2024.xlsx

| Sheet Name | Description |
| --- | --- |
| Estimation Model 1: SICT_Piers_FAF | Original FAF estimates for SICT (baseline) |
| Estimation Model 2: SICT_Piers_byPortTons | Scaled to actual port tonnage |
| Estimation Model 3: SICT_Piers_byPortTons_Liquid | Scaled excluding Liquid-Bulk |

### Liquid-Bulk Consideration

SICT wharfage data may not capture petroleum products

Liquid-Bulk typically transported via pipeline, not bridge

Alternative output retains original FAF values for petroleum

Accounts for uncertainty in data coverage

## Results

### Pier Capacity Proportions

Honolulu Harbor pier capacity allocation by cargo type:

| Pier | Container | RO/RO | Break-Bulk | Liquid-Bulk | Dry-Bulk |
| --- | --- | --- | --- | --- | --- |
| 1 (1A, 1B) | 5% | 24% | 16% | 0% | 0% |
| 2 (2A, 2B, 2C) | 0% | 0.5% | 0% | 0% | 0% |
| 19 & 20 | 0% | 9.5% | 0% | 0% | 0% |
| 29 | 5% | 0.5% | 18% | 0% | 0% |
| 30 | 0% | 0% | 0% | 58% | 0% |
| 31, 32, 33 | 0% | 44% | 0% | 0% | 0% |
| 34 | 0% | 0.5% | 0% | 0% | 0% |
| 39 & 40 | 13% | 8% | 56% | 0% | 0% |
| 51, 52, 53 (SICT) | 77% | 13% | 10% | 42% | 0% |
| 60 | 0% | 0% | 0% | 0% | 100% |
| Other Piers Total | 23% | 87% | 90% | 58% | 100% |

Key Insight: SICT dominates container handling (77%) and handles significant liquid-bulk (42%).

### SICT Share of Honolulu Harbor (FAF Model)

Overall SICT Share:

| Metric | SICT | Honolulu Total | SICT Share |
| --- | --- | --- | --- |
| Tonnage | 941,195 | 2,614,746 | 36.0% |
| Value | $6.1B | $12.5B | 48.7% |

Top 5 Commodities at SICT (by share):

| Commodity | SICT Share |
| --- | --- |
| Gasoline | 42% |
| Crude petroleum | 42% |
| Fuel oils | 42% |
| Nonmetal min. prods. | 87% |
| Mixed freight | 77% |

### Top Commodities: FAF Model

By Tonnage:

| Commodity | Tons | % of SICT |
| --- | --- | --- |
| Gasoline | 340,379 | 36.2% |
| Crude petroleum | 252,024 | 26.8% |
| Fuel oils | 217,038 | 23.1% |
| Natural gas/fossil | 23,054 | 2.4% |
| Nonmetal min. prods. | 22,110 | 2.3% |

By Value:

| Commodity | Value | % of SICT |
| --- | --- | --- |
| Gasoline | $1.11B | 18.1% |
| Crude petroleum | $408M | 6.7% |
| Mixed freight | $1.27B | 20.8% |
| Other foodstuffs | $605M | 9.9% |
| Furniture | $435M | 7.1% |

### Top Commodities: Scaled Model

Key Insight: After calibration, petroleum products (86% of FAF tonnage) are no longer dominant. Manufactured goods, mixed freight, and food products emerge as the top commodities — these are the goods most likely transported by truck over the Sand Island Access Road bridge.

By Tonnage (scaled to actual port data):

| Commodity | Scaled Tons | % of SICT |
| --- | --- | --- |
| Nonmetal min. prods. | 537,225 | 13.2% |
| Mixed freight | 499,603 | 12.2% |
| Paper articles | 393,651 | 9.6% |
| Articles-base metal | 314,447 | 7.7% |
| Milled grain prods. | 300,350 | 7.3% |

By Value (scaled):

| Commodity | Scaled Value | % of SICT |
| --- | --- | --- |
| Mixed freight | $6.27B | 20.8% |
| Other foodstuffs | $2.98B | 9.9% |
| Furniture | $2.15B | 7.1% |
| Meat/seafood | $1.57B | 5.2% |
| Paper articles | $1.37B | 4.6% |

### Top Commodities: Liquid-Bulk Excluded

By Tonnage (Liquid-Bulk retains FAF values):

| Commodity | Tons | % of SICT |
| --- | --- | --- |
| Nonmetal min. prods. | 537,225 | 11.1% |
| Gasoline | 340,379 | 7.0% |
| Mixed freight | 499,603 | 10.3% |
| Paper articles | 393,651 | 8.1% |
| Articles-base metal | 314,447 | 6.5% |

Key Insight: When Liquid-Bulk is excluded from scaling, petroleum products (Gasoline, Crude petroleum, Fuel oils) retain original FAF estimates since they may be transported via pipeline rather than truck.

## Conclusion

### Key Takeaways

Combined FAF regional data with actual port data for robust estimates

Preserved FAF commodity detail while matching observed throughput

Produced pier-level allocations based on operational capacity

Created multiple output scenarios to address data uncertainty

SICT handles 36% of Honolulu Harbor tonnage and 49% of value

After calibration, non-petroleum goods (mixed freight, food products, construction materials) dominate — directly relevant to bridge truck traffic

### Limitations & Considerations

| Limitation | Impact | Mitigation |
| --- | --- | --- |
| FAF underestimates SICT throughput (~941K vs ~4.1M tons) | Scaling factors are large | Calibrated with actual wharfage data |
| Temporal mismatch: FAF CY2024 vs. wharfage FY2025 | Minor seasonal variation possible | One full year assumed representative |
| Liquid-Bulk may be excluded from wharfage reports | Petroleum tonnage uncertain | Separate output scenario retains FAF values |
| Scaling assumes FAF commodity proportions hold at actual volumes | Commodity mix may differ | Best available approach given data constraints |

### Output Files

| File | Contents |
| --- | --- |
| FAF_Hawaii_Region_2024.xlsx | 3 estimation model sheets (SICT_Piers_FAF, SICT_Piers_byPortTons, SICT_Piers_byPortTons_Liquid) |
| SICT_Analysis_Results.xlsx | Summary statistics: pier proportions, SICT share, top commodities |